# SOS Emergency Alert System

ESP32 + Blynk IoT Platform — Complete Project Documentation

# 1. Project Overview

This project implements a wireless SOS Emergency Alert System using an ESP32 microcontroller and the Blynk IoT cloud platform. When a physical SOS button is pressed, the ESP32 sends an alert to the Blynk dashboard over Wi-Fi, displaying the button status in real time via a red LED widget. The ESP32 is preferred over the ESP8266 for its dual-core processor, more GPIO pins, and better stability.

# 2. Components Required

## 2.1 Hardware

- ESP32 development board (e.g., ESP32 DevKit V1 / WROOM-32) — built-in Wi-Fi & Bluetooth

- Push button — momentary, normally open (NO)

- 10 kΩ resistor — pull-down for button signal pin

- LED (any colour) + 220 Ω resistor — optional external status indicator (GPIO2 has onboard LED)

- Breadboard and jumper wires

- Micro-USB cable — for power and code upload

## 2.2 Software & Libraries

- Arduino IDE 1.8+ or 2.x — download from arduino.cc

- ESP32 Board Package — add this URL in File > Preferences > Board Manager URLs: https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Blynk Library — install via Sketch > Include Library > Manage Libraries > search Blynk (by Volodymyr Shymanskyy)

- Blynk Console account — free registration at blynk.cloud

# 3. Circuit Connections

Wire the components to the ESP32 as shown below. The ESP32 operates at 3.3 V logic — do not connect 5 V signals directly to GPIO pins.

| Component Pin | ESP32 Pin | Notes |
| --- | --- | --- |
| Button — signal leg | GPIO4 | Signal pin — reads HIGH (3.3 V) when pressed |
| Button — power leg | 3.3V | Connect to ESP32 3.3 V rail |
| 10 kΩ Resistor | GPIO4 to GND | Pull-down — keeps pin LOW when button is open |
| LED anode (+) | GPIO2 (onboard) | Onboard LED; or use any GPIO with 220 Ω external LED |
| LED cathode (–) | GND | Ground rail |
| ESP32 VIN / USB | Micro-USB Power | Power via USB cable (5 V regulated to 3.3 V on board) |
| ESP32 GND | GND rail | Common ground for all components |

# 4. Blynk Cloud Setup

- Go to blynk.cloud and log in or create a free account.

- Click "+ New Template". Set Name: SOS, Hardware: ESP32, Connection: WiFi. Save.

- In the Datastreams tab, click "+ New Datastream" > Virtual Pin. Set: Name = SOS Button Status, Pin = V0, Data Type = Integer, Min = 0, Max = 1. Save.

- In the Web Dashboard tab, click Edit. Drag a LED widget onto the canvas. Click the widget gear icon and link it to Datastream V0. Save.

- Go to Devices > "+ New Device" > From Template > select SOS. Name it "SOS". Click Create.

- Copy the Auth Token displayed on screen. Also note your Wi-Fi SSID and password.

- Paste the Auth Token, SSID, and password into the Arduino sketch file (sos_blynk.ino) before uploading.

Note: The Arduino source code is provided as a separate file — sos_blynk.ino. Open it directly in the Arduino IDE.

# 5. Upload & Test

- Open sos_blynk.ino in Arduino IDE.

- Go to Tools > Board > ESP32 Arduino > ESP32 Dev Module (or your specific ESP32 board variant).

- Go to Tools > Port and select your ESP32 COM port.

- Fill in your BLYNK_AUTH_TOKEN, Wi-Fi SSID, and password in the code.

- Click Upload (Ctrl+U). If it fails, hold the BOOT button on the ESP32 during upload, release after upload starts.

- Open Serial Monitor (Ctrl+Shift+M) at 115200 baud — you should see: SOS System Ready.

- Press the SOS button — the Blynk dashboard LED widget turns red and Serial Monitor shows SOS ACTIVATED!

- Release the button — the LED returns to off and Serial Monitor shows SOS cleared.

# 6. Troubleshooting

| Problem | Solution |
| --- | --- |
| Device shows Inactive on Blynk | Check Wi-Fi SSID/password and Auth Token. Open Serial Monitor at 115200 baud for connection logs. |
| Upload fails / port not found | Hold the BOOT button on ESP32 during upload. Use a data-capable USB cable (not charge-only). |
| Button press not detected | Verify GPIO4 wiring and 10 kΩ pull-down resistor. Confirm button leg connects to 3.3 V (not 5 V). |
| Blynk library not found | Install via Library Manager. Search 'Blynk' and install by Volodymyr Shymanskyy (not the legacy version). |
| Widget not updating on dashboard | Ensure LED widget is linked to V0 datastream. Check Blynk free plan message quota (200k messages). |
| ESP32 board not found in IDE | Add ESP32 board manager URL in Preferences, then install ESP32 package via Tools > Board Manager. |

# 7. Project File Reference

| File | Description |
| --- | --- |
| sos_blynk.ino | Arduino sketch — open in Arduino IDE, fill in your credentials, and upload to ESP32. |
| SOS_Arduino_Blynk_Project.docx | This documentation — project overview, wiring, Blynk setup, upload steps, and troubleshooting. |

SOS Emergency Alert System — ESP32 + Blynk IoT | Project Documentation